Final Project Report

Steven Barringer, Lauren Ruffner, Turner Levey  GROUP 22

Problem 1. Design a minimal instruction set architecture for an 8 bit load-store processor with appropriate binary encoding. Describe the ISA in detail in the report with all your design assumptions and rationale.

Ref) Detailed description of your Instruction Set Architecture covering the following points

- A table showing the all the instructions you implemented, along with their opcodes.

- For each instruction, explain what the instruction does along with the format of the instruction in terms of bit width – such as which bits represent the opcodes, which bits represent the values being read in (if applicable).

  - The file that holds the hex code to save in the RAM is classed TEST ADD

Problem 2. Using the assembly language code that you designed in the previous step, create a program to perform the matrix addition.

Ref) Attach your Y86 assembly code, and corresponding captured output screen (generated by YIS). Clearly mention your assumption for matrix A, matrix B, and expected output matrix C.

0x0000:              	 |  Main:

0x0000: 30f001000000  | 	irmovl $1, %eax

0x0006: 30f300000000 | 	irmovl $0, %ebx

0x000c: 400300000000  | 	rmmovl %eax, 0(%ebx)

0x0012: 30f002000000  | 	irmovl $2, %eax

0x0018: 400304000000  | 	rmmovl %eax, 4(%ebx)

0x001e: 30f003000000   | 	irmovl $3, %eax

0x0024: 400308000000  | 	rmmovl %eax, 8(%ebx)

0x002a: 30f004000000   | 	irmovl $4, %eax

0x0030: 40030c000000   | 	rmmovl %eax, 12(%ebx)

|

0x0036: 30f005000000   | 	irmovl $5, %eax

0x003c: 30f300000000  | 	irmovl $0, %ebx

0x0042: 400310000000 | 	rmmovl %eax, 16(%ebx)

0x0048: 30f006000000  | 	irmovl $6, %eax

0x004e: 400314000000  | 	rmmovl %eax, 20(%ebx)

0x0054: 30f007000000  | 	irmovl $7, %eax

0x005a: 400318000000  | 	rmmovl %eax, 24(%ebx)

0x0060: 30f008000000  | 	irmovl $8, %eax

0x0066: 40031c000000  | 	rmmovl %eax, 28(%ebx)

|

|

0x006c: 500300000000.  | 	mrmovl 0(%ebx), %eax

0x0072: 501310000000   | 	mrmovl 16(%ebx), %ecx

0x0078: 6001                   | 	addl %eax, %ecx

0x007a: 401320000000   | 	rmmovl %ecx, 32(%ebx)

|

0x0080: 500304000000   | 	mrmovl 4(%ebx), %eax

0x0086: 501314000000   | 	mrmovl 20(%ebx), %ecx

0x008c: 6001                   | 	addl %eax, %ecx

0x008e: 401324000000   | 	rmmovl %ecx, 36(%ebx)

|

0x0094: 500308000000.  | 	mrmovl 8(%ebx), %eax

0x009a: 501318000000   | 	mrmovl 24(%ebx), %ecx

0x00a0: 6001                   | 	addl %eax, %ecx

0x00a2: 401328000000.  | 	rmmovl %ecx, 40(%ebx)

|

0x00a8: 50030c000000.  | 	mrmovl 12(%ebx), %eax

0x00ae: 50131c000000.  | 	mrmovl 28(%ebx), %ecx

0x00b4: 6001                   | 	addl %eax, %ecx

0x00b6: 40132c000000   | 	rmmovl %ecx, 44(%ebx)

0x00bc: 00                       | halt

|

Assumptions for A and B: We assumed each matrix was of dimension 2 by 2.

The numbers in matrix A were {1,3,5,7} respectively.  The numbers in matrix B were {2,4,6,8} respectively.  These added together to form matrix C which is {3,7,11,15} respectively.

Problem 3.  Design and verify a minimal processor that can execute the ISA in activity # 1.

Only design the bare minimum circuits which are absolutely essential to execute the program that you created in assembly, and design the peripheral input/output circuit(s) for this designed processor, which can interface with the RAM/ROM modules. You can do this by modifying the peripheral circuit which was you implemented in Lab 4. (Program/edit that ROM with the binary code that you developed in step 2, verify its correct execution on your processor.)

Ref) For each functional block of the processor.

- Explain its operation. (You may want to insert screen shots from Logisim to help you in your explanation)

  - Ram – Holds all of the hex code and stores the final output in a random location.

  - Decoder – Takes in input from the hex code and returns something that can be run in registers.

  - Registers – Hold data for use later.  We have 4 registers: eax, ebx, ecx, and edx.

  - ALU – Adds two 8-bit numbers.  It is a 8-bit carry out.  Returns valE which is then sent back into the register with an address where it should be saved in memory.

  - PC Updater – The assembly code is run through the PC updater and the value is updated.  Each line of code is run through it in this fashion.

  - Buffer Registers – The buffers hold the initial instruction until it finds everything it needs.

  - Register File – Holds the registers.  Either valA and valB is one of the outputs of the register file depending on the instructions.

- If you split the work of creating these various subblocks, clearly demarcate which parts were contributed by each of the members.

Most of the work was done in lab and divided evenly between the members.  The functionality of each of the sub-blocks was contributed to by at least two members simultaneously.

Problem 5.  Final Project Demo in lab to TA, it will be part of your grade.

Show your TA how your processor run your program of activity #2.

Done